Here is your complete **Examiner Presentation & Defense Guide**.

Follow this structure during your project evaluation to impress your teacher and demonstrate complete mastery of the network.

---

# PART 1: The 2-Minute Opening Pitch

*(Say this when the examiner says: "Explain your project.")*

> **"Good morning/afternoon, Sir/Ma'am.**

> My project is a **Hierarchical, Dual-Stack (IPv4/IPv6) Enterprise University Campus Network** designed in Cisco Packet Tracer.

>

> The network integrates:

> 1. **Core Campus:** Segmented into dedicated VLANs for Administration, Computer Science, IoT, Library, Security, and internal Web/DNS servers using **802.1Q Router-on-a-Stick** for inter-VLAN routing.

> 2. **Loop-Free Layer 2 Switching:** Running **Rapid-PVST (802.1w)** with our Core Switch configured as the Root Primary, preventing broadcast storms on redundant physical links.

> 3. **Separate Hostel & Wireless Branch:** Connected via a dedicated edge router running **DHCP Services** and an **Access Point** for WPA2-secured mobile clients.

> 4. **Dynamic Routing:** Utilizing **RIPv2 with classless routing (`no auto-summary`)**, where the Campus router dynamically propagates the Internet default route to the Hostel.

> 5. **Internet Edge & Security:** An ISP gateway providing **PAT (Port Address Translation / NAT Overload)** allowing internal private hosts to securely access external internet services.

> 6. **Full Dual-Stack IPv6:** End-to-end global unicast addressing and routing running concurrently alongside IPv4 without disruption."

---

# PART 2: Core Concepts You Must Explain

When asked about the underlying theory, explain these 6 core pillars:

### 1. VLANs & Router-on-a-Stick (802.1Q)

* **Concept:** Switches operate at Layer 2. By default, different VLANs cannot communicate because they are in separate broadcast domains.

* **Implementation:** Instead of running 7 physical cables from the switch to the router, we created virtual subinterfaces (`Gi0/1.10`, `Gi0/1.20`, etc.) on a single trunk link using `encapsulation dot1Q`. The router acts as the default gateway for each subnet.

### 2. Spanning Tree Protocol (Rapid-PVST / 802.1w)

* **Concept:** Redundant links create switching loops that cause broadcast storms and MAC table instability.

* **Implementation:** We enabled Rapid-PVST and explicitly configured `SW-CORE` as the **Root Primary** (`priority 24576`). In the triangle loop between SW-CORE, SW-ADMIN, and SW-ACADEMIC, port `Fa0/23` on `SW-ADMIN` was placed into **Alternate Blocking (`Altn BLK`)**, eliminating loops while providing instant failover.

### 3. Dynamic Routing: RIPv2 vs Static Routing

* **Concept:** Static routes require manual changes and do not adapt to topological failures. Dynamic routing protocols discover paths automatically.

* **Why RIPv2?** RIPv1 is classful and ignores subnet masks. RIPv2 supports VLSM/CIDR. We added `no auto-summary` so our `/24` subnets are not summarized into classful boundaries, and used `default-information originate` so R-HOSTEL automatically learns the gateway to the Internet.

### 4. NAT / PAT (Port Address Translation Overload)

* **Concept:** RFC 1918 private IPv4 addresses (`192.168.0.0/16`) are non-routable on the public Internet.

* **Implementation:** On `R-ISP`, we configured PAT Overload (`ip nat inside source list 1 interface Gi0/1 overload`). It translates hundreds of internal private sockets into a single public IP (`203.0.113.1`) by tracking unique Layer 4 port numbers.

### 5. DNS & Web Intranet Architecture

* **Concept:** Users do not memorize IP addresses.

* **Implementation:** `DNS-Server` (`192.168.70.11`) hosts an A-record mapping `www.campus.local` to `192.168.70.10`. Clients across all VLANs query this DNS server and seamlessly load the intranet website.

### 6. Dual-Stack IPv6 & SLAAC

* **Concept:** Dual-stack allows IPv4 and IPv6 to operate simultaneously on the same physical interfaces.

* **Implementation:** Routers forward IPv6 packets via `ipv6 unicast-routing`. Wired PCs use static global unicast addresses (`2001:db8:acad:xx::/64`), while wireless laptops obtain their prefix dynamically via SLAAC (Router Advertisements).

---

# PART 3: The 5-Step Live Demonstration Script

Perform these steps in front of the teacher. This flow is logical, fast, and covers 100% of the project.

### STEP 1: Show Loop-Free Layer 2 & STP

**On SW-CORE:**

```text

show spanning-tree summary

show spanning-tree vlan 10

```

* **Point out to teacher:** *"Sir/Ma'am, SW-CORE shows 'This bridge is the root' for all campus VLANs, and its ports are in Designated Forwarding."*

**On SW-ADMIN:**

```text

show spanning-tree vlan 10

```

* **Point out to teacher:** *"Notice that Fa0/24 is our Root port facing SW-CORE, and redundant port Fa0/23 is in Alternate Blocking (`Altn BLK`) to prevent loops."*

---

### STEP 2: Show Dynamic Routing (RIPv2)

**On R-CAMPUS:**

```text

show ip route

```

* **Point out to teacher:** *"Notice the routes marked with `R`. R-CAMPUS dynamically learned the Hostel subnets (192.168.50.0, 192.168.80.0, 192.168.199.0) from R-HOSTEL via RIPv2 with Administrative Distance 120."*

---

### STEP 3: Show Wi-Fi & DHCP

1. Open **Laptop1** → **Desktop** → **IP Configuration**.

* **Point out to teacher:** *"Laptop1 connected wirelessly to SSID `CAMPUS-WIFI` using WPA2-PSK and automatically received its IP `192.168.80.21`, default gateway `192.168.80.1`, and campus DNS server `192.168.70.11` via DHCP."*

---

### STEP 4: Demonstrate DNS, Web & Live NAT

1. Open **Admin-PC1** → **Web Browser** → Enter `http://www.campus.local`.

* **Result:** The Campus website loads.

2. Open **Admin-PC1** → **Command Prompt** → Run:

```text

ping 203.0.113.10

```

* **Result:** 100% reply from the Internet Server.

3. Immediately switch to **R-ISP CLI** and run:

```text

show ip nat translations

```

* **Point out to teacher:** *"Here is the live PAT translation. Inside local `192.168.10.11` was dynamically translated to the public interface `203.0.113.1` to access `203.0.113.10`."*

---

### STEP 5: Demonstrate IPv6 Dual-Stack

1. On **R-CAMPUS**, run:

```text

show ipv6 route

```

2. On **Admin-PC1**, open Command Prompt and run:

```text

ping 2001:db8:acad:70::10

ping 2001:db8:acad:50::11

```

* **Point out to teacher:** *"Our network successfully pinged the Web Server and the Hostel PC over native IPv6 across routers."*

---

# PART 4: Top 7 Examiner Viva Questions & Winning Answers

### Q1: Why did you use Router-on-a-Stick instead of a Layer 3 Switch?

> **Answer:** *"Router-on-a-Stick is a cost-effective architectural pattern for small-to-medium campus networks. It allows multiple logical subnets to share a single physical uplink trunk using 802.1Q encapsulation tags, reducing hardware costs while providing central security policy enforcement."*

### Q2: Why did you have to enter `no auto-summary` in RIPv2?

> **Answer:** *"By default, RIP automatically summarizes subnets to their classful network boundaries (e.g., summarizing `/24` subnets into classful `/16` or `/8`). Because our campus uses contiguous classless `/24` subnets under the `192.168.0.0` space, `no auto-summary` is required so the router advertises the specific subnet masks."*

### Q3: What is the difference between Inside Local, Inside Global, Outside Local, and Outside Global?

> **Answer:**

> * **Inside Local:** The private IP assigned to an internal host (e.g., `192.168.10.11`).

> * **Inside Global:** The public IP representing the internal host to the outside world (e.g., `203.0.113.1`).

> * **Outside Local:** The IP of the external host as seen by inside devices (e.g., `203.0.113.10`).

> * **Outside Global:** The real public IP of the external destination server (e.g., `203.0.113.10`).

### Q4: Why did one link in the triangle switch topology block?

> **Answer:** *"In the loop formed by SW-CORE, SW-ADMIN, and SW-ACADEMIC, STP chose SW-CORE as the Root Bridge because of its lower bridge priority (24576). Both SW-ADMIN and SW-ACADEMIC chose their direct link to SW-CORE as their Root Port. On the redundant link between them, SW-ACADEMIC had a lower MAC address, so its port became Designated Forwarding, forcing SW-ADMIN's port (`Fa0/23`) into Alternate Blocking to break the switching loop."*

### Q5: Why was default-information originate needed on R-CAMPUS?

> **Answer:** *"R-HOSTEL only connects to R-CAMPUS. Instead of manually maintaining static default routes on R-HOSTEL, `default-information originate` causes R-CAMPUS to inject its static default route (`0.0.0.0/0`) into its RIP updates. This allows R-HOSTEL to dynamically forward all unknown Internet traffic to R-CAMPUS."*

### Q6: How do the wireless laptops get an IPv6 address without DHCPv6?

> **Answer:** *"They use SLAAC (Stateless Address Autoconfiguration). When `ipv6 unicast-routing` is enabled on R-HOSTEL, it periodically sends ICMPv6 Router Advertisement (RA) messages on `Gi0/1.80`. The laptops use this advertised `/64` prefix and generate their own host ID."*

### Q7: If routing works, why do we need NAT?

> **Answer:** *"Routing only decides the path from source to destination. However, the addresses used inside our campus (`192.168.x.x`) are private RFC 1918 addresses. Public Internet routers drop private addresses. NAT is mandatory to translate private internal IPs into globally routable public IPs."*

---

# Quick Command Reference Card (Keep this handy)

| Objective | Device | Command |

| :--- | :--- | :--- |

| **Verify STP Root** | `SW-CORE` | `show spanning-tree summary` |

| **Verify Blocked Port** | `SW-ADMIN` | `show spanning-tree vlan 10` |

| **Verify Trunks** | Any Switch | `show interfaces trunk` |

| **Verify RIP Routes** | `R-CAMPUS` / `R-HOSTEL` | `show ip route` |

| **Verify RIP Protocol**| `R-CAMPUS` / `R-HOSTEL` | `show ip protocols` |

| **Verify Live NAT** | `R-ISP` | `show ip nat translations` |

| **Verify IPv6 Routes** | Any Router | `show ipv6 route` |

| **Verify IPv6 Ports** | Any Router | `show ipv6 interface brief` |

Deliver this with confidence—your project is rock-solid and verified at every layer!